Coding in Arduino:

Basic elements of the code:

HC-SR04 Ultrasonic Sensor

Arduino

NewPing Library

HC-SR04 Ultrasonic Sensor:

Consists of a Trigger pin and an Echo pin.

Trigger Pin: Emits an ultrasonic pulse when activated.

Echo Pin: Listens for the echo, which is the reflected sound wave.

Basic overview of the code:

The distance travelled (that is, the distance between the water surface and the top of the ultrasonic sensor) is measured by sending a ping into the container. Microcontroller then processes the data received after each ping, and converts it into distance.

#include <NewPing.h>: Includes the NewPing library, which simplifies the handling of the HC-SR04 sensor.

#define TRIGGER_PIN 12: Defines pin 12 as the Trigger Pin.

#define ECHO_PIN 10: Defines pin 10 as the Echo Pin

NewPing ultrasonic(TRIGGER_PIN, ECHO_PIN);

- NewPing ultrasonic(TRIGGER_PIN, ECHO_PIN): Creates an instance of the NewPing object called ultrasonic with the trigger and echo pins.

- This instance will be used to measure distance.

float containerHeight = 22.5;  // Example: Total height of the container in cm

- float containerHeight: Represents the total height of the container in centimeters. This is a constant, which will be used to calculate the water level inside the container.

Setup Function:

void setup() {

Serial.begin(9600);  Initialize Serial Monitor

}

- Serial.begin(9600): Initializes the Serial Monitor at a baud rate of 9600, allowing the program to output data.

Loop Function:

void loop() {

delay(50);

unsigned int uS = ultrasonic.ping();

}

- delay(50): Short delay to avoid too frequent measurements.

- ultrasonic.ping(): Sends a pulse to measure the distance to the water surface. Returns the time in microseconds for the sound to travel to the surface and back.

Depth Calculation:

float waterDepth = containerHeight - uS / US_ROUNDTRIP_CM;

- US_ROUNDTRIP_CM: A constant that converts the time in microseconds to the distance in centimeters.

- The formula containerHeight - (uS / US_ROUNDTRIP_CM) calculates the depth of the water by subtracting the measured distance from the total container height.

Ensuring Valid Depth:

if (waterDepth < 0) {

waterDepth = 0;

}

- This step ensures that if the measured distance is greater than the container height, the depth is set to 0, avoiding negative values.

Output Depth:

Serial.print("Water Depth: ");

Serial.print(containerHeight - waterDepth, 2);

Serial.println(" cm");

- The calculated water depth is printed to the Serial Monitor for user observation.

- Serial.print and Serial.println are used to display the output.

- The 2 in Serial.print(containerHeight - waterDepth, 2) specifies the number of decimal places.